Blood Pressure Under Anesthesia

Submitted by Lori Fuehrer, CVT

- Monitoring blood pressure provides anesthetist with important information concerning circulatory tissue perfusion within body

- Three types of blood pressure are commonly monitored

  - Systolic Blood Pressure:

    - Represents pressure generated at ventricular depolarization

    - Normal ranges for dog and cat are: 100 - 160mm/Hg.

  - Diastolic Blood Pressure:

    - Represents pressure generated at ventricular repolarization

    - Normal ranges for dog and cat are: 60 - 100mm/Hg.

  - Mean Arterial Pressure:

    - Represents average pressure of systolic and diastolic

    - Usually about 20mm/Hg less than the systolic pressure

    - Normal ranges for dog and cat are: 80 - 120mm/Hg.

- Adequate blood pressure is important in maintaining cerebral and cardiovascular perfusion.

  - Systolic blood pressure < 80mm/Hg will not provide adequate perfusion

  - Mean blood pressure < 50mm/Hg will not provide adequate perfusion

Non-Invasive Blood Pressure Monitoring:

- Obtained by sphygmomanometer and appropriately sized cuff wrapped around an extremity of patient

- Determining the appropriate size cuff:

  - Lay cuff parallel on leg. Cuff should cover approximately 40% of leg. (Should be able to see a little bit of leg on either side of cuff.)

    - When cuff is too small BP my read > normal

    - When cuff is too large BP may read < normal

Direct Arterial Blood Pressure Monitoring:

- Obtained by arterial catheter and appropriate monitoring devices

- Method is much more reliable than non-invasive blood pressure monitoring

- Zero level of transducer should be placed at level of heart

- Dorsal metatarsal or femoral arteries are most common arteries to place catheters in for direct blood pressure readings

- Pulse wave is conducted through catheter, appropriate tubing, and then through transducer, which displays a continuous reading on machine

Hypotension:

- Defined as low blood pressure - Mean arterial pressure < 60mm/Hg.

- Causes of hypotension:

  - Hypovolemia.

- Anesthetic agents:

  - All inhalant anesthetics

  - Acepromazine (the most potent vasodilator)

  - All other agents that may cause a decrease in cardiac output or vasodilation

- Peripheral vasodilation - may be due to:

  - Anesthetic agents

  - Specific vasodilators

  - Sepsis

  - Hyperthermia

  - Hypoxia

- Cardiac disease causing reduction in cardiac output - may be due to:

  - Severe bradycardia or tachycardia

  - Tamponade

  - Myocardial disease

- Other cardiac dysrhythmias

  - Shock

  - Septic or endotoxic shock

  - Anaphylactic shock

  - Dehydration

- Treatment of hypotension:

  - Most commonly involves crystalloid, colloid, or blood product fluid administration to expand blood volume

  - Assess, and possibly lessen depth of anesthesia

  - Dobutamine:

    - Improves cardiac contractility and peripheral blood flow

    - 5 - 15mcg/kg/minute IV. Start with lower dosage, and move up as needed

  - Dopamine:

    - Low doses increase renal blood flow.

    - 1 - 3mcg/kg/minute IV

  - Higher doses will increase cardiac contractility and heart rate.

    - 3 - 7mg/kg/minute IV

  - High doses should be used with caution, as they have been associated with vasoconstriction and tachycardia.

    - 8 - 10mg/kg/minute IV

Hypovolemia:

- Defined as low circulation of blood volume.

- Hypovolemia may occur due to:

  - Severe blood or fluid loss

  - Vasodilation

  - Tachycardia

  - Vomiting

  - Diarrhea

  - Inadequate fluid intake

  - Diuresis

  - Hypoproteinemia

  - Severe dehydration

- Treatment of hypovolemia may include:

  - Fluid therapy with crystalloids or colloids

  - Appropriate blood product replacement

Hypertension:

- Defined as higher than normal blood pressure

  - Systolic pressure > 180mm/Hg.

  - Diastolic pressure > 110mm/Hg.

- Causes of hypertension:

  - Underlying disease processes such as:

    - Renal disease

    - Cardiac disease

    - Hypoadrenocorticism

    - Hyperthyroidism

    - Increased intra-cranial pressure

  - Malignant hyperthermia.

  - Hypercarbia.

  - Anesthetic drugs such as:

    - Ketamine

    - Xylazine

    - Medetomidine

  - Pain

- Treatment of hypertension:

  - Assess, and possibly increase the depth of anesthesia

  - Control ventilation to maintain a normal or low CO2 concentration

  - Administer analgesic agents

  - Discontinue use of Dopamine and Dobutamine

  - Specific treatment for malignant hyperthermia