Dental Radiography

Sara Sharp, CVT, Vickie Byard, CVT
Robin Del Bove, RVT, LVT

WHY RADIOGRAPH?: Radiography is an important part of the complete care of the dental patient.

- Many different conditions can be seen on a radiograph that may not otherwise be detected.

- Do not have to use a dental radiograph machine to take good dental films

  - The key is in using small intra-oral film.

DENTAL RADIOGRAPH MACHINES: A dental radiograph machine costs @ the same as an anesthetic machine

- Greatly enhances the dental service

- Maintenance is minimal

- Cost of radiograph film and chemicals is nominal

- Can be used to radiograph rodent, bird, kitten and puppy extremities.

- Patient positional aids increase safety.

  - Sand bags or foam pads for positioning the patient.

  - Head should be elevated and not hanging down below the animal's normal lateral plane. (helps keep the endotracheal tube from kinking).

  - Area of interest is small (minimal scatter radiation).

- Film is propped in the mouth by commercial mouth props or by gauze pads.

- Patient should always be anesthetized when dental radiographs are being taken

  - Minimizes exposure to the technician

  - Keep hands away from film and primary beam

- All dental radiograph machines must be inspected and licensed according to state law

FILM POSITIONING: Proper positioning of the film in the patient's mouth is important.

- Film should lay flat in the mouth and not be bent or curved.

  - Bent film causes distortion and may interfere with diagnostic interpretation.

- The dimple imprinted through all layers at the corner of the film should always be facing the area that you are radiographing.

- The raised dot should face the radiograph machine

  - If the film is facing in the wrong direction, the primary beam shoots through the protective lead backing = a very light film and a “honeycomb” or “tractor track” effect on the processed film.

SETTING RADIOGRAPH FILM:

- It is very important to know the anatomy of the mouth.

- Face the film in the radiograph mount with the raised dot facing towards you.

- Position the mount as if you are facing the animal and looking into the mouth of an animal facing you.

  - The animal's left side will be seen on the right side of the film

  - Mandibular projection = incisors at the bottom of the film, canines pointing up, the lower right canine will be on the left side of the film.

  - The lower right arcade will have the teeth pointing up with the first premolar on the right and the last premolar to the left.

  - Repeat this for the molars.

  - Maxilla rostral projection = incisors at the top of the film pointing downward and the right canine on the left side of the film.

  - The premolars number smallest premolar to the left of the film and the largest to the right.

  - Repeat this with the molars.

SURVEY RADIOGRAPHS

- Survey films should be done yearly.

- Radiograph only a few teeth = better detail.

- Include the rostral maxilla and mandible, and all the premolars and molars.

- Dental pathology cannot be seen by the naked eye.

- Diseases that can be detected are:

  - Periapical abscess of unbroken, but dead teeth (due to trauma.)

    - Teeth that appear healthy on the exterior, or slightly damaged/discolored, but have extensive periodontal or endodontic pathology.

  - Resorptive lesions; internally or externally beneath the gum line.

  - Teeth may appear and probe normally but still be infected or painful to the patient.

- Periodontal disease can be detected on radiograph (bone recedes from infection.)

    - The amount of bone loss on radiograph may help determine the prognosis and treatment plan for that particular tooth.

  - Detect a fracture beneath the gum line

  - Status of symphysial instability can be detected with a radiograph.

  - Root structure of multi-rooted teeth (hooked, splayed or dramatically close to the ventral mandibular cortical bone margin) *important to know prior to any extractions.

  - Tumors (most oral tumors are malignant.)

POST -OPERATIVE RADIOGRAPH

- Determines what was done and to refer to in the future.

- It is important to document that the animal left the veterinary facility with an intact jawbone.

- Radiograph a tooth socket after an extraction to detect potential problems (retained root tips, sclerotic bone, or iatrogenic fractures that occurred during the extraction process.)

BISECTING ANGLES: *Important technique to use when positioning the beam in relation to the patient and the film. This is a much better technique to use than the traditional parallel technique used on the rest of the body.

- Know your anatomy.

- Determine the plane of the tooth root, the plane of the film and estimate an imaginary line half the distance between these two planes.

- Position the primary beam of the radiograph 90 degrees to the imaginary line at the level of the tooth root to get a diagnostic film with minimal elongation or fore -shortening of the tooth roots.

  - Remember, on the upper fourth premolar, there are three roots.

    - The palatial root and cranial root are right next to each other.

    - Angle the beam about 45 degrees cranially or caudally to split the 2 roots on your radiograph.

      - Caution: the dental foramen is often confused with possible pathology. It lies over the cranial root of the second premolar in most patients.

- Advantage of the bisecting angle = periodic follow up of pathology or treatment produces a similar image, thus will comparable previous radiographs and dental disease can be accurately followed.

PROCESSING

Manual (chair side dark room)

- Inexpensive and easy to maintain

- Pay attention to film image as it is developing.

- Newer chemicals = faster development and fixing of radiographs.

  - Rule of thumb is thirty dips in the developer, wash vigorously for thirty seconds

  - Fix the film for ten minutes to retain archival quality (storage 30 years without losing detail).

  - After fixing, wash the film for an additional ten minutes before hanging them up to dry.

  - Make sure films are completely dry before mounting and filing them.

- Dental radiographs qualify as medical records

  - Should be saved for three years along with the regular medical record. Check your state's record laws.

- Automatic dental radiograph processor - follow manufacturer's instructions.

SAFETY

The dental radiograph machine is a relatively safe machine to operate as long as certain safety precautions are followed:

- Never place your hand, fingers, or any other body part in the direct beam of the machine.

- Always wear your dosimeter badge so that any scatter radiation can be monitored.

- Stand at least six feet to the side and away from the machine while it is in use.

- Wear protective shielding if you cannot be six feet away form the machine.

- Never stand in the line of the radiograph.

- Patients should always be anesthetized for dental radiographs.

  - The slightest movement will blur the images and lessen the quality of the radiograph.

  - No one should try to personally restrain an awake patient while the radiograph is being taken.

  - Patients can bite through the film and possibly swallow it.

STORAGE of FILM:

- Remember to store your unexposed radiograph film in a safe place away from direct exposure.

- Do not keep unexposed films lying next to or on the patient while taking radiographs (fogging by scatter radiation)

Rev092402